# K1X / NCNN INT8 FASTEST BACKEND CONSTITUTION

Revision V4 — RVA22-first rework for Banana-Pi BPI-F3 / SpacemiT K1X

Supersedes V3 for performance engineering. V4 keeps the old rv64gcv_zvfh lane only as a comparison/CI lane and moves the fastest standard lane to RVA22U64 + V + Zfh + K1 standard extras.

Constitutional decision Да, V3 был слишком консервативен для fastest backend. Новая ревизия принимает стандартный профильно-ориентированный путь: для максимальной скорости на K1X надо проектировать backend под полный RVA22U64-профиль, дополнять его опциями V/Zfh и использовать доступные на K1 стандартные расширения сверх профиля — прежде всего Zicond и Zbc. IME/XSMTVDot остаётся отдельным opt-in lane и не смешивается с базовым стандартным lane.

# 1. Executive verdict

K1/X60 по официальному datasheet одновременно заявлен как 64GCVB-compatible core и как процессор, соответствующий RVA22 standard. Поэтому fastest INT8 backend больше не должен ограничиваться только contract-строкой rv64gcv_zvfh, если цель — максимальная скорость именно на K1X, а не переносимость на произвольный RVV-SoC.

При этом важно исправить одну неточность из предыдущих обсуждений: Zicond сам по себе не является частью RVA22. Он стандартизован отдельно, а для K1 он важен как доступное на железе дополнительное расширение сверх базового RVA22-профиля. Поэтому V4 закрепляет не «голый RVA22», а профиль RVA22U64 + optional V/Zfh + K1 standard extras.

Главное следствие: outer dataflow проекта не меняется — narrow input staging, stem-special entry, internal packN graph, IGEMM-family for standard 3x3, direct depthwise, fused epilogue, direct-write layout ops и late tiny float tail остаются. Меняется build contract, scalar assist plane, allocator policy, cpuset-scoped feature probing и набор обязательных low-level helpers.

# 2. Constitutional divergence from the canonical project environment

| Lane | Build contract | Purpose | Status in V4 |
| --- | --- | --- | --- |
| B-CMP | -march=rv64gcv_zvfh -mabi=lp64d | Comparison lane, CI, regression guard, direct continuity with k1x-env-overview.md | Retained but demoted |
| B-RVA22-FAST | -march=rva22u64_v_zfh_zvfh_zicond_zbc -mabi=lp64d -mtune=spacemit-x60 | Primary fastest standard lane for K1X-only backend work | New constitutional default |
| B-IME-FAST | B-RVA22-FAST + XSMTVDot-enabled toolchain/asm path | Optional cluster0-only speed lane for pointwise/standard conv kernels | Opt-in acceleration lane |

Пояснение. Локальный environment contract проекта по-прежнему фиксирует K1_ARCH_FLAGS как -march=rv64gcv_zvfh -mabi=lp64d. V4 не переписывает этот canonical document задним числом, а поверх него вводит специальный performance constitution для fastest backend branch.

# 3. Revised ISA exploitation matrix

| Extension / group | Origin | How V4 uses it | Where it matters most | V4 status |
| --- | --- | --- | --- | --- |
| Zba | RVA22 mandatory | sh1add/sh2add/sh3add in scalar address generation for row-pointer setup, panel stepping, concat destination stepping | K-120 / K-130, K-160, allocator glue | Mandatory |
| Zbb + Zbs | RVA22 mandatory | scalar assist for masks, branchless clamp glue, bit tests, compact tail logic, NMS/tiny-tail helpers | K-150, K-190, runtime glue | Mandatory |
| Zicond | K1 standard extra | czero.eqz / czero.nez for branchless scalar selects and tail policies; never sold as 'RVA22 itself' | K-150 tails, K-190 late tail, runtime policy | Mandatory on K1 fast lane |
| Zic64b | RVA22 mandatory | 64-byte cache-block alignment becomes allocator law for packed slabs, scratch and skip slices | Arenas, slabs, direct-write concat destinations | Mandatory |
| Zicbop | RVA22 mandatory | prefetch.r / prefetch.w become explicit, measurable hints in hot kernels and layout kernels | K-100, K-120, K-160, K-180 | Default-on if supported by compiler |
| Zicboz | RVA22 mandatory | cbo.zero for aligned bulk zero of scratch / output / skip recycle slabs after runtime/OS enablement check | allocator, K-160 destination recycle, workspace init | Guarded mandatory path |
| Zicbom | RVA22 mandatory | never in hot inner loops; only controlled maintenance hooks for benchmarking/allocator experiments when OS enables them | allocator / diagnostics | Guarded, non-hotpath |
| Zihintpause | RVA22 mandatory | pause in worker-queue spin waits and barrier hot spots | threadpool/runtime | Mandatory |
| Zicntr + Zihpm | RVA22 mandatory | counter-backed microbench and QA probes when permitted | measurement harness | Mandatory for instrumentation design |
| V + Zfh + Zvfh | RVA22 options + standard ext | vectorized INT8 graph, fp16 carve-outs, fp16 late-tail support, mixed-precision escape hatch | whole graph | Mandatory for fastest lane |
| Zbc | K1 standard extra | low-priority scalar assist; useful but not a top-ROI kernel driver | runtime helpers, bit utilities | Enabled, secondary |
| XSMTVDot | K1 non-standard IME subset | replaces inner primitive of K-095/K-110/K-130 only; cluster0-only | IME lane | Separate opt-in lane |

# 4. Revised algorithm and dataflow

V4 не отменяет решения V3 по алгоритмам свёртки. Наоборот: полный RVA22 укрепляет именно тот hybrid dataflow, который уже был выбран как fastest candidate для K1X. 1x1 остаётся direct packed GEMM/IGEMM. Standard 3x3 остаётся IGEMM/indirect family с scalar row-pointer indirection. Depthwise 3x3 остаётся direct spatial. INT8 Winograd по-прежнему запрещён в phase 1 и phase 2.

Главное изменение здесь — ввод отдельного ISA-assist plane. Теперь runtime начинается не просто с 'load graph -> prepack -> run', а с cpuset-aware feature discovery, выбора build lane, настройки 64B-aligned arena, включения policy для prefetch/zero instructions и только потом перехода к normal graph execution.

Step A. cpuset-scoped probe: сначала определяется целевой cpuset (cluster0-only для IME, comparison lane или full standard lane для RVV path), затем считываются common standard features и cache-block policy.

Step B. narrow preprocess: raw image never becomes frame-wide pack32. Quantize boundary produces only pack3/pack4 narrow staging.

Step C. stem-special low-C entry: first packN tensor is born only inside K-090 / K-095.

Step D. packed INT8 graph: producer kernels write packed outputs directly through fused epilogues; split/slice stay view-like; concat writes into final destination.

Step E. late tiny tail: only small decode/NMS tensors may leave packed INT8. Branchless scalar tail code may use Zicond/Zbb/Zbs.

# 5. Cache, allocator and cache-block operations constitution

| Memory class | Typical residents | V4 constitutional policy | Acceleration hooks |
| --- | --- | --- | --- |
| M-A | per-thread A/B panels, row pointers, INT32 partials | always L1-scoped; 64B-aligned; no lifetime beyond current tile | prefetch.r on next panels |
| M-B | recent outputs, hot packed weight slabs | cluster-local near-term reuse class; 64B slab geometry is mandatory | prefetch.r/w, optional selective zero |
| M-C | long-lived skip buffers | default assumption at 640 is still DDR-backed explicit ownership; do not lie about residency | promote selected slices, not whole tensors |
| M-D | selected cluster0 hot slabs/slices in TCM | allocator hooks designed from day one; runtime promotion remains selective and cluster0-only | TCM promotion after stable non-TCM path |

New constitutional cache-block rule: because RVA22 mandates Zic64b, every hot packed slab, direct-write concat destination, prepack buffer and scratch arena block is treated as 64-byte aligned and 64-byte stepped by default. Tail shapes may be smaller logically, but physical slab geometry remains 64B-clean.

New constitutional CBO rule: prefetch.r / prefetch.w are part of the normal optimization toolbox and should be measured, not feared. cbo.zero is allowed only behind a runtime/OS enablement gate; it is excellent for aligned zero of scratch/output slabs, but cannot be assumed safe blindly. Zicbom maintenance operations stay out of the hot inner loop and live only in diagnostic / allocator / experiment paths.

# 6. Revised kernel family constitution

| ID | Family | V4 algorithm / contract | Key ISA assists | Priority |
| --- | --- | --- | --- | --- |
| K-000 | Feature discovery | cpuset-aware standard-feature probe + IME capability gate | hwprobe, cpuset logic | P0 |
| K-005 | Arena / slab init | 64B-aligned allocator, slab classes, zero/prefetch policy objects | Zic64b, Zicboz | P0 |
| K-010 | Input quantization | narrow pack3/pack4 staging only | Zbb/Zbs scalar glue | P0 |
| K-020 / K-030 | Weight prepack | offline packing for 1x1 and standard 3x3 families | 64B slab geometry | P0 |
| K-050 | RVA22 memory assists | prefetch and zero helpers used by producer/layout kernels | Zicbop, Zicboz | P1 |
| K-090 / K-095 | Stem-special low-C | pack3/4 -> packN 3x3 s2 entry family | Zba assist / optional XSMTVDot | P1/P2 |
| K-100 / K-110 | 1x1 pointwise | direct packed GEMM/IGEMM | prefetch, Zba outer glue, optional XSMTVDot | P1/P2 |
| K-120 / K-130 | Standard 3x3 | IGEMM/indirect family with scalar row-pointer indirection | Zba address generation, prefetch, optional XSMTVDot | P1/P2 |
| K-140 | Depthwise 3x3 | direct spatial convolution | 64B slab policy, prefetch | P1 |
| K-150 | Fused epilogue | INT32 -> bias -> requant -> PWL SiLU -> packed INT8 store | Zicond + Zbb/Zbs scalar tail assists | P1 |
| K-160 / K-170 / K-180 | Layout ops | direct-write concat, view-like split, pack-preserving upsample | Zba stepping, prefetch.w, optional cbo.zero for recycle | P1 |
| K-190 | Detect tail | keep branch convs INT8; dequant only for tiny late tensors before decode/NMS | Zicond + Zbb/Zbs in scalar tail helpers | P1 |

# 7. Revised build and runtime rules

R-BLD-001. Fastest standard lane uses B-RVA22-FAST by default. Old rv64gcv_zvfh lane survives only as B-CMP.

R-BLD-002. If the compiler supports profiles, V4 prefers profile syntax over long explicit ISA strings.

R-BLD-003. -mtune=spacemit-x60 is mandatory for K1X-targeted performance builds whenever supported by the compiler.

R-BLD-004. cpuset-scoped feature discovery is mandatory before backend dispatch. Standard features are probed on the target CPU set; IME is probed separately on cluster0-only execution.

R-BLD-005. IME stays opt-in and cluster0-only. There is no silent IME use in B-RVA22-FAST binaries.

R-ISA-001. 64-byte cache-block geometry is an allocator law, not a tuning suggestion.

R-ISA-002. Scalar address generation in standard 3x3 and layout kernels must be written to exploit Zba patterns explicitly.

R-ISA-003. Zicond is reserved for scalar tails, branchless policy code and NMS/decode helpers; it is not advertised as a vector-convolution primitive.

R-CCH-001. prefetch.r / prefetch.w belong in measurement-driven kernels and layout ops; they are not optional philosophy.

R-CCH-002. cbo.zero is allowed only behind a runtime safety gate; maintenance CBOs stay out of hot loops.

# 8. Measurement and validation

Benchmark discipline from the canonical project environment remains unchanged: cluster0-pinned 4-thread execution is the primary optimization target, 320-sized workloads additionally require a 2-thread auxiliary check, and microbench, layerbench and full-frame forward must be reported separately.

New V4 validation axis: every benchmark record must state which lane was active — B-CMP, B-RVA22-FAST or B-IME-FAST — and whether prefetch / cbo.zero policies were enabled. Without that, results are not constitutionally comparable.

# 9. Implementation roadmap

| Phase | What gets built | Why this order is constitutional |
| --- | --- | --- |
| P0 | K-000/K-005, cpuset probe, 64B arena, source registry, counters, feature traces | Without runtime observability and allocator law, the new ISA contract cannot be enforced. |
| P1 | K-010, K-020/K-030, K-050, K-090, K-100, K-120, K-140, K-150, K-160..K-190 on B-RVA22-FAST | This creates the first real end-to-end fastest standard lane with RVA22 assists active. |
| P2 | K-095, K-110, K-130 and IME dispatch on cluster0 | IME accelerates an already-correct backend instead of defining correctness itself. |
| P3 | Selective TCM promotion and allocator heuristics for skip slices / hot slabs | TCM becomes a measured accelerator, not a hidden phase-1 dependency. |

# Appendix A. Canonical build strings

Primary fastest standard lane:

-march=rva22u64_v_zfh_zvfh_zicond_zbc -mabi=lp64d -mtune=spacemit-x60

Comparison / continuity lane:

-march=rv64gcv_zvfh -mabi=lp64d

IME lane conceptually equals:

B-RVA22-FAST + XSMTVDot-enabled assembler/compiler path (toolchain-specific flag spelling)

# Appendix B. Source registry

| Source ID | Kind | URL / Path | Role in V4 |
| --- | --- | --- | --- |
| SRC-L01 | Local | /mnt/data/k1x-env-overview.md | Canonical project environment and benchmark contract; now treated as comparison-lane baseline. |
| SRC-L02 | Local | /mnt/data/NCNN_FP16_RISCV_NOTES.md | Project-local ncnn/RVV notes and canonical harness. |
| SRC-L03 | Local | /mnt/data/YOLO_NCNN_NOTES.md | Local YOLO11/K1X ground truth and hotspot observations. |
| SRC-W21 | Web | https://docs.banana-pi.org/en/BPI-F3/SpacemiT_K1_datasheet | K1 hardware facts: 64GCVB, RVA22, caches, TCM, cluster0 AI capability. |
| SRC-W22 | Web | https://docs.riscv.org/reference/profiles/rva20-rvi20-rva22/_attachments/RISC-V_Profiles.pdf | RVA22 mandatory and optional extension set, incl. Zic64b/Zicbop/Zicboz/Zihintpause/Zfhmin. |
| SRC-W23 | Web | https://gcc.gnu.org/onlinedocs/gcc/RISC-V-Options.html | GCC profile syntax and spacemit-x60 tuning target. |
| SRC-W24 | Web | https://docs.kernel.org/arch/riscv/hwprobe.html | cpuset-scoped feature discovery for standard extensions and cache-block information. |
| SRC-W25 | Web | https://docs.riscv.org/reference/isa/unpriv/cmo.html | Semantics of prefetch.r/prefetch.w and cbo.zero. |
| SRC-W26 | Web | https://docs.riscv.org/reference/isa/unpriv/zicond.html | Zicond instruction semantics. |
| SRC-W27 | Web | https://docs.riscv.org/reference/isa/unpriv/b-st-ext.html | B extension / Zba-Zbb-Zbs semantics. |
| SRC-W28 | Web | https://lists.llvm.org/pipermail/cfe-commits/Week-of-Mon-20250901/787067.html | K1 ruapu-based march string showing standard extras such as Zbc and Zicond. |
| SRC-W29 | Web | https://llvm.org/docs/RISCVUsage.html | XSMTVDot naming and scope for SpacemiT IME subset. |
| SRC-W30 | Web | https://arxiv.org/abs/1907.02129 | Indirect convolution / IGEMM algorithmic reference. |
| SRC-W31 | Web | https://ncnn.readthedocs.io/en/latest/how-to-use-and-FAQ/quantized-int8-inference.html | ncnn INT8/PTQ semantics and mixed-precision carve-outs. |

One-sentence constitutional summary

Fastest standard K1X INT8 backend = cpuset-aware RVA22-first build lane + narrow pack3/pack4 input staging + stem-special entry + internal packN graph + direct 1x1 GEMM/IGEMM + indirect/implicit 3x3 with Zba-driven scalar row pointers + direct depthwise + fused PWL epilogue + 64B-aligned slabs + measured prefetch/zero assists + late tiny float tail only.